# ELEC 2FH4 – Bonus Sets 16, 17, 18

# Set 16

Analytical Work:

MATLAB Code:

%% Set 16 - Toroid Magnetic Field Plot % Exercise: Toroid with N=200 turns, I=5.0 A, inner radius=1.5 cm, % outer radius=2.5 cm. Plot B field in x-y plane from -4 cm to 4 cm. clc; clear; close all; % Constants mu0 = 4*pi*1e-7; % permeability of free space (H/m) N = 200; % number of turns I = 5.0; % current (A) a = 0.015; % inner radius (m) b = 0.025; % outer radius (m) % Precompute constant factor B_factor = mu0 * N * I / (2*pi); % = 2e-4 % Define plotting grid (in meters) x_min = -0.04; x_max = 0.04; y_min = -0.04; y_max = 0.04; % Number of plotting points num_points_x = 25; num_points_y = 25; % Create grid x = linspace(x_min, x_max, num_points_x); y = linspace(y_min, y_max, num_points_y); [X, Y] = meshgrid(x, y); % Initialize field components Bx = zeros(size(X)); By = zeros(size(X)); % Calculate magnetic field at each point for i = 1:num_points_x for j = 1:num_points_y rho = sqrt(X(j,i)^2 + Y(j,i)^2); if rho > a && rho < b % Inside toroid core - B field present B_mag = B_factor / rho; Bx(j,i) = -B_mag * Y(j,i) / rho; % Bx = -B * sin(phi) By(j,i) = B_mag * X(j,i) / rho; % By = B * cos(phi) else % Outside toroid core - B field approximately zero Bx(j,i) = 0; By(j,i) = 0; end end end % Plot the magnetic field figure(1); quiver(X, Y, Bx, By, 'b', 'LineWidth', 1); hold on; % Draw the toroid boundaries theta_circle = linspace(0, 2*pi, 200); x_inner = a * cos(theta_circle); y_inner = a * sin(theta_circle); x_outer = b * cos(theta_circle); y_outer = b * sin(theta_circle); plot(x_inner, y_inner, 'r--', 'LineWidth', 2); plot(x_outer, y_outer, 'r--', 'LineWidth', 2); % Fill the toroid region fill([x_inner, fliplr(x_outer)], [y_inner, fliplr(y_outer)], [0.9 0.9 0.9], 'EdgeColor', 'none'); hold off; % Formatting xlabel('x (m)'); ylabel('y (m)'); title(sprintf('Magnetic Field of Toroid (N=%d, I=%.1f A)', N, I)); axis equal; axis([x_min x_max y_min y_max]); grid on; legend('B Field', 'Toroid Boundaries', 'Location', 'best'); % Add text annotation text(0.03, 0.035, sprintf('Inner radius = %.2f cm\na = %.2f cm', a*100, a*100), 'FontSize', 9, 'BackgroundColor', 'w'); text(0.03, 0.03, sprintf('Outer radius = %.2f cm\nb = %.2f cm', b*100, b*100), 'FontSize', 9, 'BackgroundColor', 'w'); text(0.03, 0.025, sprintf('N = %d turns\nI = %.1f A', N, I), 'FontSize', 9, 'BackgroundColor', 'w'); % Create a second plot showing magnitude as colormap figure(2); B_magnitude = sqrt(Bx.^2 + By.^2); surf(X, Y, B_magnitude); xlabel('x (m)'); ylabel('y (m)'); zlabel('¦B¦ (T)'); title('Magnetic Field Magnitude Distribution'); colorbar; colormap('jet'); view(45, 30); grid on; disp('Set 16 - Toroid Magnetic Field Plot Completed');

MATLAB Output:

# Set 17

Analytical Work:

MATLAB Code:

%% Set 17 - Mutual Inductance of Two Coaxial Coils % Exercise: Two coils with radii a1=0.01 m, a2=0.04 m, separated by d=0.1 m. % Find mutual inductance. clc; clear; close all; % Constants mu0 = 4*pi*1e-7; % permeability of free space (H/m) % Given parameters a1 = 0.01; % radius of first coil (m) a2 = 0.04; % radius of second coil (m) d = 0.1; % separation distance between coils (m) N1 = 1; % number of turns in first coil (assuming 1 turn) N2 = 1; % number of turns in second coil (assuming 1 turn) % Method 1: Using the exact formula with elliptic integrals k = sqrt(4*a1*a2 / ((a1 + a2)^2 + d^2)); % Complete elliptic integrals using MATLAB's built-in functions [K, E] = ellipke(k^2); % Neumann's formula for mutual inductance of two coaxial circular loops M_exact = mu0 * sqrt(a1*a2) * ((2/k - k)*K - (2/k)*E); % Multiply by number of turns if more than 1 M_exact = M_exact * N1 * N2; % Method 2: Using approximation for small coil (a1 << a2, d) % B field from large coil at its center: B = mu0 * N2 * I / (2*a2) % For small coil at distance d along axis, B = mu0 * N2 * I * a2^2 / (2*(a2^2 + d^2)^(3/2)) % Then M = B * (pi*a1^2) / I M_approx = mu0 * pi * a1^2 * a2^2 * N1 * N2 / (2 * (a2^2 + d^2)^(3/2)); % Method 3: Numerical integration using Biot-Savart law % Discretize both coils into segments num_segments = 200; M_numerical = mutual_inductance_numerical(a1, a2, d, mu0, N1, N2, num_segments); % Display results fprintf('\n========================================\n'); fprintf('SET 17: Mutual Inductance Calculation\n'); fprintf('========================================\n'); fprintf('Parameters:\n'); fprintf(' a1 = %.4f m (%.2f cm)\n', a1, a1*100); fprintf(' a2 = %.4f m (%.2f cm)\n', a2, a2*100); fprintf(' d = %.4f m (%.2f cm)\n', d, d*100); fprintf(' N1 = %d, N2 = %d\n', N1, N2); fprintf(' mu0 = %.2e H/m\n', mu0); fprintf('\nResults:\n'); fprintf(' Method 1 (Exact - Elliptic Integrals): M = %.4e H\n', M_exact); fprintf(' Method 2 (Approximation): M = %.4e H\n', M_approx); fprintf(' Method 3 (Numerical Integration): M = %.4e H\n', M_numerical); fprintf('\n'); % Plot the configuration figure(1); clf; % Plot coil 1 (small coil) at z = -d/2 theta = linspace(0, 2*pi, 100); x1 = a1 * cos(theta); y1 = a1 * sin(theta); z1 = -d/2 * ones(size(theta)); % Plot coil 2 (large coil) at z = d/2 x2 = a2 * cos(theta); y2 = a2 * sin(theta); z2 = d/2 * ones(size(theta)); % 3D plot subplot(1,2,1); plot3(x1, y1, z1, 'b-', 'LineWidth', 2); hold on; plot3(x2, y2, z2, 'r-', 'LineWidth', 2); plot3([0 0], [0 0], [-d/2 d/2], 'k--', 'LineWidth', 1); xlabel('x (m)'); ylabel('y (m)'); zlabel('z (m)'); title('Coaxial Coil Configuration'); legend('Coil 1 (a=0.01m)', 'Coil 2 (a=0.04m)', 'Axis'); grid on; axis equal; view(45, 30); % 2D projection subplot(1,2,2); plot(x1, y1, 'b-', 'LineWidth', 2); hold on; plot(x2, y2, 'r-', 'LineWidth', 2); plot(0, 0, 'ko', 'MarkerSize', 8); xlabel('x (m)'); ylabel('y (m)'); title('Top View (x-y plane)'); legend('Coil 1', 'Coil 2', 'Center'); axis equal; grid on; sgtitle('Mutual Inductance Problem Geometry'); % Function for numerical integration function M = mutual_inductance_numerical(a1, a2, d, mu0, N1, N2, num_segments) % Discretize the angles phi1 = linspace(0, 2*pi, num_segments + 1); phi2 = linspace(0, 2*pi, num_segments + 1); % Coil 1 is at z = -d/2, Coil 2 is at z = d/2 z1 = -d/2; z2 = d/2; % Precompute coil points points1_x = a1 * cos(phi1(1:end-1)); points1_y = a1 * sin(phi1(1:end-1)); points1_z = z1 * ones(size(points1_x)); points2_x = a2 * cos(phi2(1:end-1)); points2_y = a2 * sin(phi2(1:end-1)); points2_z = z2 * ones(size(points2_x)); % Calculate dl vectors for coil 1 dl1_x = diff(a1 * cos(phi1)); dl1_y = diff(a1 * sin(phi1)); dl1_z = zeros(size(dl1_x)); % Calculate dl vectors for coil 2 dl2_x = diff(a2 * cos(phi2)); dl2_y = diff(a2 * sin(phi2)); dl2_z = zeros(size(dl2_x)); % Calculate mutual inductance using Neumann's formula M_sum = 0; for i = 1:num_segments for j = 1:num_segments % Position vectors r1 = [points1_x(i), points1_y(i), points1_z]; r2 = [points2_x(j), points2_y(j), points2_z]; % dl vectors dl1 = [dl1_x(i), dl1_y(i), dl1_z(i)]; dl2 = [dl2_x(j), dl2_y(j), dl2_z(j)]; % Distance between segments R_vec = r2 - r1; R = norm(R_vec); % Dot product of dl vectors dl1_dot_dl2 = dot(dl1, dl2); % Add contribution to mutual inductance M_sum = M_sum + dl1_dot_dl2 / R; end end M = mu0/(4*pi) * M_sum * N1 * N2; end fprintf('Set 17 - Mutual Inductance Calculation Completed\n');

MATLAB Output:

======================================== SET 17: Mutual Inductance Calculation ======================================== Parameters: a1 = 0.0100 m (1.00 cm) a2 = 0.0400 m (4.00 cm) d = 0.1000 m (10.00 cm) N1 = 1, N2 = 1 mu0 = 1.26e-06 H/m Results: Method 1 (Exact - Elliptic Integrals): M = 2.5011e-10 H Method 2 (Approximation): M = 2.5279e-10 H Method 3 (Numerical Integration): M = 1.1151e-13 H Set 17 - Mutual Inductance Calculation Completed

# Set 18

Analytical Work:

MATLAB Code:

%% Set 18 - Toroid Self Inductance % Exercise: Toroid with N=200 turns, inner radius=2.0 cm, outer radius=2.5 cm. % Find self inductance analytically and numerically. clc; clear; close all; % Constants mu0 = 4*pi*1e-7; % permeability of free space (H/m) % Given parameters N = 200; % number of turns a = 0.02; % inner radius (m) b = 0.025; % outer radius (m) h = b - a; % height of toroid (assuming square cross-section) (m) % Analytical solution (assuming rectangular cross-section) L_analytical = (mu0 * N^2 * h) / (2*pi) * log(b/a); % Display analytical result fprintf('\n========================================\n'); fprintf('SET 18: Toroid Self Inductance\n'); fprintf('========================================\n'); fprintf('Parameters:\n'); fprintf(' N = %d turns\n', N); fprintf(' a = %.4f m (%.2f cm)\n', a, a*100); fprintf(' b = %.4f m (%.2f cm)\n', b, b*100); fprintf(' h = %.4f m (%.2f cm)\n', h, h*100); fprintf(' mu0 = %.2e H/m\n', mu0); fprintf('\nAnalytical Result:\n'); fprintf(' L_analytical = %.4e H = %.4f μH\n', L_analytical, L_analytical*1e6); %% Numerical Solution Method 1: Energy Method % Calculate total magnetic energy stored in toroid % Energy = 1/2 * L * I^2, so L = 2*Energy/I^2 I_test = 1.0; % Test current (A) % Discretize the toroid volume num_rho = 100; % divisions in radial direction num_phi = 100; % divisions in azimuthal direction num_z = 50; % divisions in vertical direction drho = (b - a) / num_rho; dphi = 2*pi / num_phi; dz = h / num_z; total_energy = 0; for i = 1:num_rho rho = a + (i - 0.5) * drho; B = mu0 * N * I_test / (2 * pi * rho); H = B / mu0; for j = 1:num_phi for k = 1:num_z % Volume element dV = rho * drho * dphi * dz; % Energy density = 1/2 * B * H dW = 0.5 * B * H * dV; total_energy = total_energy + dW; end end end L_numerical_energy = 2 * total_energy / I_test^2; fprintf('\nNumerical Results:\n'); fprintf(' Method 1 (Energy Method): L = %.4e H = %.4f μH\n', ... L_numerical_energy, L_numerical_energy*1e6); %% Numerical Solution Method 2: Flux Linkage Method % Direct calculation of flux linkage num_phi_flux = 200; % divisions for flux calculation num_rho_flux = 200; drho_flux = (b - a) / num_rho_flux; dphi_flux = 2*pi / num_phi_flux; % Calculate flux through one turn at mean radius % For a toroid, flux through each turn is approximately the same flux_per_turn = 0; for i = 1:num_rho_flux rho = a + (i - 0.5) * drho_flux; B = mu0 * N * I_test / (2 * pi * rho); % Area element in the cross-section dA = drho_flux * h; flux_per_turn = flux_per_turn + B * dA; end % Total flux linkage lambda = N * flux_per_turn; L_numerical_flux = lambda / I_test; fprintf(' Method 2 (Flux Linkage): L = %.4e H = %.4f μH\n', ... L_numerical_flux, L_numerical_flux*1e6); %% Method 3: Using formula for circular cross-section toroid % For toroid with circular cross-section of radius r_c r_c = h / 2; % radius of circular cross-section L_circular = mu0 * N^2 * (sqrt(b) - sqrt(a))^2; fprintf(' Method 3 (Circular CS approx): L = %.4e H = %.4f μH\n', ... L_circular, L_circular*1e6); %% Plot the magnetic field distribution in the toroid figure(1); clf; % Create grid for plotting rho_plot = linspace(a, b, 50); phi_plot = linspace(0, 2*pi, 50); [rho_grid, phi_grid] = meshgrid(rho_plot, phi_plot); % Calculate B field magnitude B_mag = mu0 * N * I_test ./ (2 * pi * rho_grid); % Convert to Cartesian coordinates for plotting X = rho_grid .* cos(phi_grid); Y = rho_grid .* sin(phi_grid); % Plot B field magnitude as a surface surf(X, Y, B_mag); xlabel('x (m)'); ylabel('y (m)'); zlabel('¦B¦ (T)'); title('Magnetic Field Distribution in Toroid'); colorbar; colormap('jet'); view(0, 90); % Top view axis equal; grid on; % Add toroid boundaries hold on; theta_circle = linspace(0, 2*pi, 100); x_inner = a * cos(theta_circle); y_inner = a * sin(theta_circle); x_outer = b * cos(theta_circle); y_outer = b * sin(theta_circle); plot(x_inner, y_inner, 'k--', 'LineWidth', 2); plot(x_outer, y_outer, 'k--', 'LineWidth', 2); hold off; % Second plot: B field vs radial distance figure(2); rho_line = linspace(a, b, 200); B_line = mu0 * N * I_test ./ (2 * pi * rho_line); plot(rho_line*100, B_line*1e6, 'b-', 'LineWidth', 2); xlabel('Radial Distance (cm)'); ylabel('Magnetic Field ¦B¦ (μT)'); title('Magnetic Field Inside Toroid vs. Radial Position'); grid on; xlim([a*100, b*100]); % Add vertical lines for inner and outer boundaries hold on; yl = ylim; plot([a*100 a*100], yl, 'r--', 'LineWidth', 1.5); plot([b*100 b*100], yl, 'r--', 'LineWidth', 1.5); text(a*100, yl(2)*0.9, sprintf('a = %.1f cm', a*100), ... 'HorizontalAlignment', 'right', 'Color', 'r'); text(b*100, yl(2)*0.9, sprintf('b = %.1f cm', b*100), ... 'HorizontalAlignment', 'left', 'Color', 'r'); hold off; %% Summary Table fprintf('\n========================================\n'); fprintf('SUMMARY - Self Inductance of Toroid\n'); fprintf('========================================\n'); fprintf('Method ¦ Inductance (H) ¦ Inductance (μH)\n'); fprintf('---------------------------¦-------------------¦-----------------\n'); fprintf('Analytical (rectangular) ¦ %.4e ¦ %.4f\n', L_analytical, L_analytical*1e6); fprintf('Numerical (energy method) ¦ %.4e ¦ %.4f\n', L_numerical_energy, L_numerical_energy*1e6); fprintf('Numerical (flux method) ¦ %.4e ¦ %.4f\n', L_numerical_flux, L_numerical_flux*1e6); fprintf('Circular cross-section ¦ %.4e ¦ %.4f\n', L_circular, L_circular*1e6); fprintf('========================================\n'); % Calculate percent difference between methods diff_percent = abs(L_numerical_energy - L_analytical) / L_analytical * 100; fprintf('\nPercent difference (Analytical vs Energy Method): %.2f%%\n', diff_percent); disp('Set 18 - Toroid Self Inductance Calculation Completed');

MATLAB Output:

======================================== SET 18: Toroid Self Inductance ======================================== Parameters: N = 200 turns a = 0.0200 m (2.00 cm) b = 0.0250 m (2.50 cm) h = 0.0050 m (0.50 cm) mu0 = 1.26e-06 H/m Analytical Result: L_analytical = 8.9257e-06 H = 8.9257 μH Numerical Results: Method 1 (Energy Method): L = 8.9257e-06 H = 8.9257 μH Method 2 (Flux Linkage): L = 8.9257e-06 H = 8.9257 μH Method 3 (Circular CS approx): L = 1.4006e-05 H = 14.0060 μH ======================================== SUMMARY - Self Inductance of Toroid ======================================== Method ¦ Inductance (H) ¦ Inductance (μH) ---------------------------¦-------------------¦----------------- Analytical (rectangular) ¦ 8.9257e-06 ¦ 8.9257 Numerical (energy method) ¦ 8.9257e-06 ¦ 8.9257 Numerical (flux method) ¦ 8.9257e-06 ¦ 8.9257 Circular cross-section ¦ 1.4006e-05 ¦ 14.0060 ======================================== Percent difference (Analytical vs Energy Method): 0.00% Set 18 - Toroid Self Inductance Calculation Completed